Int. J. Electrochem. Sci., 10 (2015) 4495 - 4512

International Journal of

ELECTROCHEMICAL

SCIENCE

www.electrochemsci.org

Enhancement of an Electrocoagulation Process for the Treatment of Textile Wastewater under Combined Electrical Connections Using Titanium Plates

Ahmed Samir Naje1,  Shreeshivadasan Chelliapan2,*,  Zuriati Zakaria2, Saad A. Abbas2

1Department of Environmental Science, Collage of Sciences, Al-Muthana University, Iraq.

2Department of  Environmental Engineering and Green Technology, Malaysia-Japan International Institute of Technology (MJIIT), Universiti Teknologi Malaysia, Jalan Semarak, 54100, Kuala Lumpur, Malaysia.

*E-mail: shreeshivadasan.kl@utm.my

Received: 27 February 2015/  Accepted: 2 April 2015  /  Published: 28 April 2015

The performance of an electrocoagulation (EC) process by adding titanium plates (electrooxidation, EO) was studied for the treatment of textile wastewater using combined electrical connections in a single reactor. Aluminum (and iron) and Titanium (Ti) electrodes were fixed in a bipolar (Bp), and monopolar (Mp) structure in the same electrolytic cell. The performance of the reactor was characterised in terms of electrolysis time (RT), current intensity (I), pH, chemical support, inter-electrode distance (de), and stirring speed (Mrpm). Furthermore, energy and electrode consumption, sludge compaction, operating expenses as well as comparison with the conventional EC process was also examined. The most suitable EC-EO performance was achieved by using Mp Ti-Bp Al plates. Preliminary results showed the following optimal operating conditions: I=0.6 A, pH= 6, de=1cm  and Mrpm=500 rpm. The implementation of these parameters on textile wastewater revealed a relatively high removal efficiency ofCOD (93.5%), TSS (97%), color (97.5%), BOD5 (90%), TDS (89%), turbidity (96%), phenols (99%) and phosphate (97%). The overall operating cost for the process was 1.69 US$/m3. This value was calculated based on the electrode and energy consumption, chemicals, and sludge disposal.